A Model for Pavement Design and Management Based on The AASHTO Guide and HDM Empirical Models

Paper submitted to the Transportation Research Record

Prof. Régis Martins Rodrigues (Aeronautics Technological Institute, ITA)

ABSTRACT

This paper describes the technical base of the performance prediction models which are applied on a pavement management system developed for Brazilian highways, whose main feature was to reduce to a minimum a major shortcoming of existing systems: the usual great discrepancy between maintenance solutions and costs established for planning purposes at network level and the ones that are effectively detected after studies are made at the project level. This was accomplished through close attention to the pavement structure-traffic interaction aspect of the performance prediction models. For this purpose, a generalization of the AASHTO Guide model was developed, introducing long-term pavement performance effects by World Bank HDM empirical models. Calibration factors for the performance prediction models were determined for several Brazilian highway networks and found to be consistent with the varying environmental and construction conditions encountered.

# 1 - Introduction

At network level, when a Pavement Management System applies performance prediction models, usually they are of a simplistic nature, due to the necessity of keeping monitoring field data at low cost per kilometer. Besides, several systems attempt only to detect, at one side, the pavement segments where minor maintenance measures can be applied, and at the other side, the segments where a major rehabilitation should be scheduled. In these versions of a PMS there are no need for truly reliable performance prediction models at network level.

On the other hand, experience with a PMS applied to several highway networks in Brazil led to the need for performance models that go further: they should have the ability to anticipate structural needs that will not be grossly over- or underestimated when rehabilitation designs are performed. A complete agreement between network and project level solutions is impossible, since additional factors must be taken into account at project level, such as localized environmental conditions (drainage, geometry, traffic loads and lateral distribution), pavement history and detailed pavement condition and layer materials data. However, when managerial characteristics impose the need for a good agreement between these two levels of decision, the performance models to be applied at network level must have adequate accuracy to deal properly with structural parameters (traffic loads and their interaction with pavement structure). This is particularly important in Brazil due to the broad range covered by the bearing capacities of highway pavements, with major extensions of highways operating, due to budget constraints, with relatively thin pavements under heavy traffic loading conditions.

In fact, applications of HDM system in Brazil have been deemed to failure due to misconceptions on its use. Instead of running the program only for pre-selected strategies, generated after sound technical considerations have been made regarding pavement performance, strategies were implemented based solely on least initial cost and the resulting expected benefits due to vehicle operation costs decrease. This led, for example, to thin resurfacings applied over severely cracked pavements without anti-reflection cracking interlayers, or to thin overlays applied to structures experiencing bad performance due to excessive plastic strains developing in the layers below the surfacing layer. Besides, lack of a proper consideration regarding traffic-structure interaction by the merely empirical correlations on HDM led to false expectations about rehabilitated pavement service lives under the use of these thin overlays. Such experience highlight the importance of a PMS with sound performance prediction models and reliable decision trees for the detections of pavement maintenance needs.

The PMS here described has been in use for over 12 years, mainly for the following purposes:

- For budged estimates in the near future;

- As an aid in rehabilitation designs at project level, furnishing diagnoses,  maintenance solutions and pavement and traffic data;

- On special studies, so that ones involving simulations of different performance patterns required for the pavements on the network and the costs associated with the maintenance of these patterns;

- As a tool for designs made before an actual design at the project level is conducted, so that a fast and reasonable estimate of rehabilitation needs can be obtained, as well as to avoid gross mistakes at project level.

Pavement construction and its subsequent maintenance involve consumption of precious natural resources and of energy. To avoid a wasteful use of these resources is a central task for any PMS, which require truly reliable pavement performance prediction models in order to optimize the allocation of limited funds to the purpose of maintaining an adequate service level for the highway users.

The final purpose of a PMS is to allow the discovery of the most effective pavement maintenance strategy, for which a measure of the comparative efficacy of different strategies is required. A central parameter to be maximized on a pavement construction or rehabilitation strategy is the Benefit-cost Ratio (BCR) associated with the investment, which can be defined as:

BCR=PSIavCeq

[1]

where PSIav is the average serviceability and Ceq is the equivalent uniform annual maintenance cost, associated with the investment during its service life (Lf), or:

PSIav=1Lf0LfPSI(t)dtCeq=COSTLf

[2]

Such a definition for BCR allows the comparison to be made of different design periods (Lf) and different standard levels for pavement performance (PSI0 and PSIt on Figure 1) so that the most efficient solution can be found.

The achievement of a certain pavement performance is not, however, a certainty, due to the stochastic nature of pavement behavior. Each design has its proper reliability level (RL), which represents the probability of the outcome expressed by PSIav. Therefore, the best solution is the one that maximizes the Expected Utility, defined by:

EU=prU=RLBCR=RLPSIavCeq

[3]

The maximization of this parameter involves the application of reliable models for the prediction of pavement performance in relation to its reliability level.

Figure 1 – Serviceability with time for a pavement investment

# 2 – Development of the Performance Prediction Models

Based on the previous discussion, the following characteristics were judged useful for the performance prediction models:

- The models must be able to describe the long-term performance of highway pavements;

- The interaction between pavement structure and traffic loads must be accurately dealt with;

- Inputs for the model should be the data usually available from network monitoring programs;

- The models should be easily calibrated, so that remaining life estimates could be done using pavement past performance;

- Pavement condition should be adequately and easily described, as well as its modification through pavement maintenance or rehabilitation;

- Due consideration must be given to the stochastic nature of pavement performance, so that a desired reliability level for the performance predictions can be adopted as a fundamental parameter for the conception of a maintenance strategy.

Existing performance prediction models aimed at pavement management applications, such as HDM models (Paterson, 1987), have limited applicability due to their empirical nature and close association with a limited set of field data. They have, however, the advantage of being able to describe long-term pavement performance. On the other side, design models, such as the AASHTO Design Guide (1986), are good at describing traffic-structure interaction, but are not compatible to long-term pavement performance effects, due to the accelerated nature of the AASHO Road Test. Therefore, the procedure followed here for the development of the performance prediction models was to link these two powerful models, avoiding their shortcomings and using their strengths.

The present condition of a pavement can be describe in a complete manner by the use of three indicators:

- A serviceability parameter, related to user riding comfort and vehicle operation costs;

- A pavement distress index, useful as a gauge to anticipate pavement maintenance needs and to keep the risks of a sudden pavement failure under control;

- Security related indexes, so that the ones related to directional control (hydroplaning potential), breaking distance (friction on wet surfaces) and obstacles (potholes, surfacing disintegration).

The third set of indicators should be addressed through proper network monitoring of the pavement surface condition, whereas the first two are affected by traffic-structure interaction, so that they will be the ones addressed in this paper for development of performance prediction models.

The first parameter can be the AASHTO Present Serviceability Index (PSI). Use of the PSI has the advantages of being linked to a sound model the traffic-structure interaction (the AASHTO Guide for Design of Pavement Structures) and to its close relationship with riding comfort, as expressed by

PSI=5e−QI71.5

[4]

where QI is the Quarter-Car Index, in counts per km, that measures longitudinal roughness (QI = 13×IRI, where IRI is the International Roughness Index, m/km). This equation was derived in a World Bank experimental study (Paterson, 1987).

The second parameter should be a function of the nature, extent and severity of pavement distresses, as given by a Surface Condition Index (SCI), which will be defined here on the same scale of the PSI, from 0 to 5, and with the same meaning regarding their respective levels, as shown in Table 1.

Table 1 – Scale for the PSI and the SCI

| Range | Pavement Condition |
| --- | --- |
| 4 – 5 | Excellent |
| 3 – 4 | Good |
| 2 – 3 | Regular |
| 1 – 2 | Bad |
| 0 – 1 | Very bad / Pavement failure |

For this purpose, the SCI was determined from a relationship with a global distress index known as the IGG (Pereira, 1979), given by:

SCI=309,22−0,616×IGG61,844+IGG

IGG=i=1nfipi

[5]

The IGG is thus determined from the nature, extent and severity of the existing surface distresses, as the summation of the products of the corresponding frequency (fi) versus weighing factor (pi) of the n existing defects. Table 2 shows these weighing factors as used in Brazil. A close relationship exists between this parameter and the USACE PCI: PCI = -2.7E-6 * (IGG ^ 3) + 0.0025 * (IGG ^ 2) - 0.8109 * IGG + 100.

Table 2 – Weighing factors for the IGG (Pereira, 1979)

| Distress Type | fi  pi |
| --- | --- |
| FC-1, TRR | fi  0.2 |
| D | fi  0.3 |
| Average Rutting | 1.33  Rutting in mm (≤ 30 mm) |
| FC-2, TB | fi  0.5 |
| Rutting Variance | Variance (≤ 50 mm2) |
| EX | fi  0.5 |
| R | fi  0.6 |
| FC-3, TBE | fi  0.8 |
| ALP, ATP | fi  0.9 |
| P, O | fi  1 |

A model for roughness progression can be written in terms of traffic, expressed as the number of accumulated standard 80 kN axle loads N until pavement age t is reached, as:

IRI(N)=IRI0eαN

[6]

where  is a function of the pavement structural strength and of the environmental conditions (climate, geometry, drainage). In this model, the product N expresses the traffic-structure interaction, which controls the increase of IRI with time. IRI0 is an intercept at pavement age equal to zero and is nearly equal to the initial pavement roughness. This functional shape is supported by several pavement performance data (see, for example, Figure 1) and empirical models, such as (Paterson, 1987):

IRIt=IRI0+725FC1+SNC−4,99Nte0,0153t

[7]

where:

FC = model calibration factor;

IRI(t) = International Roughness Index at year t (m/km);

t   = pavement age since construction or last rehabilitation (years);

N(t) = total 80 kN axle load accumulated  number of repetitions, in millions;

IRI0 = initial pavement longitudinal roughness (m/km).

Figure 1 - Section SHRP_ID = 1, STATE_CODE = 48 (DataPave 3.0)

Merging equations [4] and [6] leads to:

PSI(N)=5PSI05expαPSIN

[8]

with PSI0 being the initial pavement serviceability index. The functional shape of equation [8] has all the desirable features surrounding parameter PSI evolution with time, as revealed by long-term pavement performance data. Besides, a hypothesis will be launched here involving the practical applicability of writing:

SCI(N)=5SCI05expαSCIN

[9]

where SCI0 is the initial (new or newly rehabilitated pavement) Surface Condition Index. Up to this point, nothing can prove the validity of equation [9], but as will be shown here its calibration to several pavement data bases has been possible with an acceptable degree of accuracy.

In equations [8] and [9], if the structural parameters PSI and SCI are known, then future performance of the pavement can be predicted. In this respect, emphasis must be given to the determination of how these  values vary with pavement structural parameters.

The AASHTO Design Guide (1986) model can be applied in the context of determining parameter PSI, since it was derived from a test especially designed to evaluate the traffic-structure interaction.  The AASHTO model is not, however, reliable as a performance prediction model, due to the accelerated nature of the AASHO Road Test, where long-term effects could not be evaluated. Therefore, this model will be here applied exclusively with the objective of arriving at a nominal pavement traffic bearing capacity, leading to a first estimate of parameter , capable of including in the prediction process the key variables related to pavement structure and traffic loads. Long-term second order effects will be taken into account after these key effects are included in the model. In this respect, the AASHTO design model will be here applied in the same context as it is intended for in The AASHTO Guide (1986): to furnish an estimate of the accumulated traffic that is required for a pavement global serviceability loss from a new pavement condition (PSI0) until pavement “failure” (PSIf), when a rehabilitation, such as a major asphalt concrete overlay, will be needed. Therefore, the first estimate of parameter , as given by the AASHTO Guide, will be:

αA=−1W18lnlnPSI05ln2.55

[10]

where W18 is the 18 kip single axle loads accumulated number of repetitions associated with a condition change from PSI0 to PSIf = 2.5. According to the 1986-1993 AASHTO Design Guide:

W18=10ZRS0SN+11.05129.36PSI0−2.54.2−1.51βMR3000psi2.32β=0.40+1094SN+15.19

[11]

where SN (=  aihi) is the pavement Structural Number, MR is the subgrade resilient modulus (psi) and PSI0 is the initial Present Serviceability Index.

With the most important variables already included in the model by this process, it is time now to deal with long-term effects, environmental conditions and other factors not included through the use of equations [10] and [11]. This will be done by defining a calibration factor FC applied to parameter , or:

αPSI=Fc×αA

[12]

A major effort in this research is at establishing the functional dependence of FC on its controlling variables. The performance prediction model here proposed can be considered reliable only if patterns can be identified of FC variation with environmental (climatic, geometric, drainage) and construction (materials, construction quality and construction processes) conditions. Besides, FC on equation [12] must be less than one, due to the gradual increase with time of materials strength (tixotropy effects) that would be observed on a pavement without traffic.

# 3 – Calibration from HDM Models

The HDM-III models (Paterson, 1987) have several shortcomings as true performance prediction models, especially when considering its narrow experimental base and lack of mechanistic considerations, both leading to loss of predicting power. The effects of a change in thickness of the asphalt concrete surfacing layer are not properly dealt with, since its increase only slightly increases pavement service life. Therefore, these models were here applied only for a selected set of “reference conditions”, chosen to represent the experimental data base. It is believed that, for such conditions, the models predictions are not biased and will reflect the life expectancy for those conditions.

Determination of FC will be made for a group of pavement sections for which field long-term performance is known. HDM empirical models, as described by Paterson (1987), will be applied for this purpose, involving only a selected set of highway pavement sections, the ones situated inside the universe of the sections that were used to develop such models. This requirement is a need dictated from the purely empirical nature of HDM models and from the fact that they employ only few data parameters for the consideration of traffic loading and pavement structure effects on pavement performance. This results in the models being too simplistic to reliably describe reality, as well as leads them to suffer from a fundamental lack of accuracy to consider the effects of variations of pavement materials mechanical properties and of pavement layers thicknesses on pavement performance. Therefore, HDM models can only be trusted as a source of reliable long-term pavement performance data for pavement sections with similar characteristics of the ones that comprised the data base that was used in the regression analyses that led to those models. These were pavements situated on Brazilian highways, which were designed using the DNER Method (DNIT, 2006), a version of the original USACE-CBR method. In this method, total pavement thickness in terms of granular materials is given by:

HT(cm)=9,02+0,23log10NP+0,057011CBR−234,3312

[13]

for the pavement to support NP equivalent 80 kN single axle load repetitions for a subgrade of bearing capacity equal to CBR. Besides, Table 3 shows the minimum surfacing layer thickness required by the method and this thickness must be converted to equivalent granular layer thickness using a coefficient equal to 2, so that: HT = 2h1 + h2 + h3 for the total pavement thickness on a section with surfacing (h1), base (h2) and sub-base (h3) layers.

Table 3 – Minimum surfacing layer requirement on DNER Method (DNIT, 2006)

| NP | Surfacing Layer |
| --- | --- |
| ≤ 106 | Surface treatments |
| 106 < NP ≤ 5 106 | 5 cm of asphalt concrete |
| 5  106 < NP ≤ 107 | 7,5 cm of asphalt concrete |
| 107 < NP ≤ 5 107 | 10 cm of asphalt concrete |
| NP > 5  107 | 12,5 cm of asphalt concrete |

The resilient moduli of the subgrade soil will be given by: MR = 10  CBR (MPa). For granular sub-base materials: MR = 5  CBR (MPa), and for the granular base, the Shell formula was applied, as given by:

E2E3=0,2×h20,45

[14]

for base thickness (h2) in mm.

It was also required that the selected pavement sections met a maximum deflection under the 80 kN axle load as expressed by: Dadm (0.01 mm) = 10(3.01-0.176logNp), which is also a part of DNER method for pavement design and evaluation.

Considering a set of pavement sections that were generated according to DNER Design Method, Table 4 shows the resulting calibration factors required in order that the model here proposed could reproduce pavement performance as described through HDM empirical models. Three values for FC were calculated, each corresponding to the pavement layer with resilient modulus MR above which the structural number SN was computed for use on equation [11]. In spite of the variability, it can readily be seen that the average values of FC are representative enough of the eleven selected cases to be taken as the calibration factors to be applied for pavement design and analysis. Besides, as predicted, FC < 1 for PSI prediction, due to the lack of long-term effects affecting materials strength and stiffness at the AASHO Road Test.

Table 4 – Results for PSI prediction on new asphalt pavements

| Case | Fc_Subgrade | Fc_Subbase | Fc_Base |
| --- | --- | --- | --- |
| 1 | 0.920 | 0.689 | 0.161 |
| 2 | 0.980 | 0.694 | 0.205 |
| 3 | 0.970 | 0.599 | 0.219 |
| 4 | 0.850 | 0.449 | 0.105 |
| 5 | 0.950 | 0.633 | 0.113 |
| 6 | 0.730 | 0.405 | 0.097 |
| 7 | 0.940 | 0.461 | 0.137 |
| 8 | 0.970 | 0.420 | 0.091 |
| 9 | 0.790 | 0.440 | 0.127 |
| 10 | 0.880 | 0.623 | 0.127 |
| 11 | 0.950 | 0.542 | 0.180 |
| 12 | 0.870 | 0.630 | 0.181 |
| 13 | 1.400 | 0.563 | 0.122 |
| 14 | 1.000 | 0.554 | 0.129 |
| 15 | 1.500 | 0.544 | 0.101 |
| 16 | 1.030 | 0.432 | 0.103 |
| Average = | 0.983 | 0.542 | 0.137 |
| Std.Dev. = | 0.199 | 0.097 | 0.040 |
| S0PP = | 0.202 | 0.180 | 0.291 |

Table 4 shows that the value of FC, calculated for the whole pavement structure above the subgrade, is nearly equal to 1, which implies on the validity of the AASHTO Guide as a long-term performance prediction model only if the model is applied under this kind of view. Considering the two other possibilities (the pavement consisting of the surfacing and base layers above the sub-base, and the pavement being viewed as the surfacing layer above the base layer), FC is well below one, showing the effects of accelerated nature of the AASHO Road Test.

Table 4 also shows a measure of the uncertainty regarding performance prediction using the proposed model, in terms of the ratio S0PP = StdDev / Average for FC for the several cases here analyzed. The AASHTO Guide overall standard deviation S0 has a recommended value around 0.4, which makes sense, since half this value is due to the uncertainties solely associated with the act of performance prediction, with the remaining 0.2 being due to the pavement layers variation in terms of thicknesses and materials  mechanical properties.

Table 5 and Figure 2 show these calculations for Case 5. The procedure was to find by trial and error the best FC value for the subgrade analysis. The other two FC values, corresponding to the sub-base and base layers, were simply given by: FC = /A.

Following the same procedure, Table 6 shows the resulting calibration factors for the case of SCI prediction along time. Now, FC values should not always be less than one, as in the case of PSI prediction, since the AASHTO model does not deal explicitly with pavement distresses, as described by the SCI, but only with riding comfort, as described by the PSI. Table 7 and Figure 3 illustrates for one of the cases the calculations that were done in order to arrive at these results.

Table 6 – Results for SCI prediction on new asphalt pavements

| Case | Fc_Subgrade | Fc_Subbase | Fc_Base |
| --- | --- | --- | --- |
| SCI_1 | 2.050 | 0.938 | 0.253 |
| SCI_2 | 2.450 | 1.446 | 0.670 |
| SCI_3 | 2.650 |  |  |
| SCI_4 | 2.650 | 0.861 | 0.232 |
| SCI_5 | 2.500 | 0.553 | 0.085 |
| Average = | 2.460 | 0.949 | 0.310 |

Table 5 – Analysis for Case 5 for PSI prediction

| Layer | h1 (cm) | ai | MR (kgf/cm2) | CBR |
| --- | --- | --- | --- | --- |
| Asphalt Concrete | 7 | 0.44 | 40000 |  |
| Base (Crushed Stone) | 20 | 0.14 | 3982 | 100 |
| Granular Subbase | 20 | 0.11 | 1835 | 30 |
| Subgrade |  |  | 1000 | 10 |
| PP (years) = | 12 |  | CBRp subgrade = | 7.7 |
| HTp (DNER) = | 53.9 | cm | HT (cm) = | 54 |
| Nano (AASHTO) = | 5.00E+05 |  | SN = | 3.149 |
| QI0 = | 17 |  | RH = | 0 |
| PSI0 QI = | 3.942 |  | MR (psi) = | 14224.75 |
| Np (USACE) = | 1.80E+07 |  |  |  |
| h1_min. = | 10 | cm | SNC = | 4.379 |
| VDMc = | 913 |  | TYcr2 = | 4.95 |
|  |  |  | D0_SNC mm = | 0.330 |
| (Subgrade) |  |  | Dadm = | 56.9 |
| AlfaA = | 0.135802067 |  | Vs (TR=20%) = | 9.0 |
| W18 = | 7.878902057 |  | D0_FLAPS = |  |
| Beta = | 1.079241462 |  |  |  |
| Fc = | 0.950 |  |  |  |
| Alfa = | 0.129011964 |  |  |  |
| (Subbase) |  |  |  |  |
| AlfaA = | 0.203907891 |  | SN = | 2.29 |
| W18 = | 5.247326036 |  | MR (psi) = | 26104 |
| Beta = | 2.649143494 |  |  |  |
| Fc = | 0.633 |  |  |  |
| (Base) |  |  |  |  |
| AlfaA = | 1.144421604 |  | SN = | 1.21 |
| W18 = | 0.934944938 |  | MR (psi) = | 56649 |
| Beta = | 18.14078503 |  |  |  |
| Fc = | 0.113 |  |  |  |

Figure 2 – Results for the analysis of Case 5 on PSI prediction with time

Table 7 – Analysis for Case 1 for SCI prediction

| Material | h1 (cm) | ai | MR(kgf/cm2) | CBRp |
| --- | --- | --- | --- | --- |
| CBUQ | 7.5 | 0.44 | 40000 |  |
| BG | 15 | 0.14 | 2860 | 100 |
| SBG | 15 | 0.11 | 1500 | 30 |
| SL |  |  | 1000 | 10 |
| HTP(DNER) = | 44.8 | cm | HT (cm) = | 45 |
| Nano = | 3.00E+05 |  | SN = | 2.751 |
| QI0 = | 17 |  | RH = | 0 |
| PSI0 QI = | 3.942 |  | MR (psi) = | 14225 |
| Np (USACE) = | 9.00E+06 |  |  |  |
| h1_mín. = | 7.5 | cm | SNC = | 3.981 |
| VDMc = | 548 |  | TYcr2 = | 5.30 |
|  |  |  | D0_SNC mm = | 0.713 |
| (Subleito) |  |  | Dadm = | 61.9 |
| AlfaA = | 0.919929332 |  | Vs (TR=20%) = | 9.30 |
| W18 = | 5.183168599 |  | D0_FLAPS = | 47 |
| Beta = | 1.545502152 |  |  |  |
| Fc = | 2.050 |  |  |  |
| Alfa = | 1.885855131 |  |  |  |
| (Sub-base) |  |  |  |  |
| AlfaA = | 2.010430565 |  | SN = | 2.11 |
| W18 = | 2.371705301 |  | MR (psi) = | 21337 |
| Beta = | 3.429619299 |  |  |  |
| Fc = | 0.938 |  |  |  |
| (Base) |  |  |  |  |
| AlfaA = | 7.458002706 |  | SN = | 1.30 |
| W18 = | 0.639333212 |  | MR (psi) = | 40682 |
| Beta = | 14.93522085 |  |  |  |
| Fc = | 0.253 |  |  |  |

Figure 3 – Results for the analysis of Case 1 on SCI prediction with time

In the case of a rehabilitated pavement, this same model can be applied, since such configuration resembles the performance prediction made for a new pavement considering now the pavement structure as consisting only of the surfacing layer supported by the base layer. Therefore:

- The Structural Number will now be given by: SN = a1HR, where a1 = 0.44 is the asphalt concrete overlay structural coefficient and HR is its thickness (in inches);

- MR=21−ν2paDCThe “roadbed” resilient modulus will now be given by:

[15]

where MR is calculated as the semi-infinite elastic space equivalent modulus of the pavement structure (before rehabilitation), and DC is the pavement deflection, measured under a circular applied load of pressure p and radius a. The Poisson’s ratio  of this equivalent system can be fixed at an average typical value for pavement materials (such as 0.33).

The idea behind considerations (a) and (b) is that deterioration of a rehabilitated pavement is essentially the generation of distresses inside the applied asphalt concrete overlay, with the old pavement behaving only as an elastic platform which gives a support described by its equivalent elastic modulus MR, calculated by equation [15]. Such consideration will lead to an underestimate of pavement performance only if the old pavement has a significant remaining life, since in this case it cannot be degenerated simply as an “elastic platform”. Practical applications of this model in such cases can be made by adding an effective overlay thickness to the actual applied overlay thickness HR, with this effective thickness calculated using the model for new pavements in order to reproduce its remaining service life.

It is unlikely that the value of FC for a new pavement can also be applicable to a rehabilitated one, since an existing pavement has already suffered the conditioning effects due to traffic loads, as well as long-term climatic and aging effects. It can be anticipated that FC for a new pavement must be greater than the value for an overlaid one, since the conditioning effects that the traffic loads apply to all pavement layers lead to a pavement structure more stable than a new one, which has been subjected only to the stress history applied by compaction equipment. By the other hand, a severely cracked old pavement can lead to the reflection cracking process controlling the rehabilitated pavement performance, which will increase FC values.

The procedure here adopted was to consider the overlay case with the same view of a new pavement, but using the FC value associated with the base case (FC = 0.137), since this is the situation closer to the overlay one.

Four overlay cases were here considered (Table 8), varying traffic rate (Nyear) and existing pavement conditions prior to overlay: deflection (Dc) and longitudinal roughness (QIa). HDM (Paterson, 1987) and George and Rajagopal (1989) empirical models were applied in order to arrive at the pavement performance prediction. They showed the need to correct the overlay structural number by a factor Cf = 1.555 in order for the model to reproduce PSI behavior given by HDM models (Figure 4).

Table 8 – Rehabilitation cases analyzed

| Overlay Case | Cf | Nyear (AASHTO) | Dc (0.01 mm) | QIa (counts/km) |
| --- | --- | --- | --- | --- |
| 1 | 1.710 | 1.00E+06 | 70 | 45 |
| 2 | 1.570 | 3.00E+06 | 60 | 50 |
| 3 | 1.395 | 2.00E+06 | 70 | 45 |
| 4 | 1.545 | 5.00E+05 | 80 | 50 |
| Average = | 1.555 |  |  |  |

For SCI prediction, the resulting average value for Cf was 1.254.

| Material | h1 (cm) | ai | MR (kgf/cm2) | CBR |
| --- | --- | --- | --- | --- |
| CBUQ | 6.5 | 0.44 |  |  |
| Pav. Exist. | Dc = | 80 | 1875 | 7 |
| Nano = | 5.00E+05 |  | SN = | 1.126 |
|  |  |  | RH = | 1 |
| PSI0 QI = | 3.51 |  | MR (psi) = | 26677.50 |
| QI0 = | 25.31 |  |  |  |
| QIa = | 50 |  | SNC = | 4.680 |
| VDMc = | 913 |  | TYcr2 = | 3.95 |
| Dadm = | 55.8 |  | D0 = | 55.0 |
|  |  |  | PCR4 = | 48 |
| AlfaA = | 0.631127547 |  |  |  |
| W18 = | 1.064750289 |  |  |  |
| Beta = | 6.253065721 |  |  |  |
| Fc = | 0.137 |  |  |  |
| Alpha = | 0.086464474 |  |  |  |
| Cf = | 1.545 |  |  |  |

Figure 4 – Results for the analysis of Overlay Case 1 on PSI prediction with time

# 4 – Case Studies

The model will be here applied to a set of independent data in order to see if it is capable of giving a reliable performance prediction. For new pavements, highways SP-300 (Figure 5) and SP-070 (Figure 6) will be analyzed. Both of them were not designed using DNER Method, but through mechanistic-empirical analyses.

At SP-300 (segment Itu – Jundiaí in São Paulo state), the new highway was opened to traffic in 2006 on west direction between km 92 and km 102.2 and on the east direction between km 98,2 and km 72. The average condition of the pavements in 2012 on lane 2, where commercial traffic was concentrated, was given by QI = 34.4 counts/km and IGG = 39, under a traffic rate of 4.55  105 ESAL´s (AASHTO equivalency factors) per year. Pavement deflection was at an average value of 41  10-2 mm under 40 kN (FWD), SN = 3.04 and subgrade elastic modulus was MR = 147 MPa. Pavement structure consisted of 5 cm AC surfacing layer, 16 cm granular base layer (crushed stone) and 19 cm soil-cement subbase, above a subgrade soil with CBR ≥ 13.

Figure 5 – Analysis of the new pavement on SP-300 highway

At SP-070 (segment Taubaté – Jacareí in São Paulo state), the new highway was opened to traffic in 1995 between km 61.3 and km 130.4. The average condition of the pavements in 2012 on lane 2, where commercial traffic was concentrated, was given by QI = 27 counts/km and IGG = 9 to 14, under a traffic rate of 5  105 ESAL´s (AASHTO equivalency factors) per year. Pavement deflection was at an average value of 27  10-2 mm under 40 kN (FWD), SN = 3.6 and subgrade elastic modulus was MR = 217 MPa. Pavement structure consisted of 12 cm AC surfacing layer, 15 cm granular base layer (crushed stone) and 17 cm cemented-treated crushed stone subbase, above a subgrade soil with CBR ≥ 15.

Figure 6 – Analysis of the new pavement on SP-070 highway

In the case of rehabilitated pavements, an interesting outcome was the overlay of SP-160 using 3 cm of  an asphalt-rubber concrete. Figure 7 shows the performance prediction for lanes 3 and 4 (where commercial traffic was concentrated). The pavement was overlaid in 2006. In 2013, longitudinal roughness was at QI = 13 counts/km. In 2014, it was QI = 19 counts/km, with IGG = 8. It can be seen that this performance was essentially predicted with success by the model.

Figure 7 – Overlaid pavement on SP-160 highway

A final verification of the model was performed applying it to pavement sections designed using the TRRL Road Note 29 (Croney, 1977). Table 9 shows the derived reliability levels and the pavement terminal conditions required for the pavement sections to be in agreement with the model here developed. It is apparent that the design reliability level increases and the pavement acceptable conditions during service life become more stringent as the design traffic increases, in ways similar to the ones recommended by the AASHTO Guide.

Figure 8 illustrates the procedure here adopted for the cases listed on Table 9.

Table 9 – Application of the model to Road Note 29

| Case | Np AASHTO | Nano | CBRsl | h1cm | h2cm | h3cm | RL (%) | QIt | IGGt |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.00E+04 | 5.00E+02 | 2 | 5 | 10 | 32 |  |  |  |
|  | 1.00E+04 | 5.00E+02 | 3 | 5 | 10 | 25 |  |  |  |
|  | 1.00E+04 | 5.00E+02 | 4 | 5 | 10 | 18 |  | 60 | 70 |
|  | 1.00E+04 | 5.00E+02 | 5 | 5 | 10 | 13 |  |  |  |
|  | 1.00E+04 | 5.00E+02 | 6 | 5 | 10 | 11 |  |  |  |
|  | 1.00E+04 | 5.00E+02 | 7 | 5 | 10 | 10 |  |  |  |
|  | 1.00E+05 | 5.00E+03 | 2 | 5.5 | 12 | 39 |  |  |  |
|  | 1.00E+05 | 5.00E+03 | 3 | 5.5 | 12 | 29 |  |  |  |
|  | 1.00E+05 | 5.00E+03 | 4 | 5.5 | 12 | 21 |  | 50 | 60 |
|  | 1.00E+05 | 5.00E+03 | 5 | 5.5 | 12 | 16 |  |  |  |
|  | 1.00E+05 | 5.00E+03 | 6 | 5.5 | 12 | 13 |  |  |  |
|  | 1.00E+05 | 5.00E+03 | 7 | 5.5 | 12 | 10.5 |  |  |  |
|  | 1.00E+06 | 5.00E+04 | 2 | 6.5 | 16 | 45 |  |  |  |
|  | 1.00E+06 | 5.00E+04 | 3 | 6.5 | 16 | 32 |  |  |  |
|  | 1.00E+06 | 5.00E+04 | 4 | 6.5 | 16 | 23 |  | 45 | 50 |
|  | 1.00E+06 | 5.00E+04 | 5 | 6.5 | 16 | 18 |  |  |  |
|  | 1.00E+06 | 5.00E+04 | 6 | 6.5 | 16 | 15 |  |  |  |
|  | 1.00E+06 | 5.00E+04 | 7 | 6.5 | 16 | 11 | 50 |  |  |
|  | 1.00E+07 | 5.00E+05 | 2 | 12 | 22 | 54 |  |  |  |
|  | 1.00E+07 | 5.00E+05 | 3 | 12 | 22 | 40 |  |  |  |
|  | 1.00E+07 | 5.00E+05 | 4 | 12 | 22 | 30 |  | 40 | 40 |
| S2 | 1.00E+07 | 5.00E+05 | 5 | 12 | 22 | 24 | 70 |  |  |
|  | 1.00E+07 | 5.00E+05 | 6 | 12 | 22 | 21 |  |  |  |
| S1 | 1.00E+07 | 5.00E+05 | 7 | 12 | 22 | 16 | 80 |  |  |
|  | 1.00E+08 | 5.00E+06 | 2 | 21 | 27 | 62 |  |  |  |
|  | 1.00E+08 | 5.00E+06 | 3 | 21 | 27 | 47 |  |  |  |
|  | 1.00E+08 | 5.00E+06 | 4 | 21 | 27 | 37 | 85 | 35 | 30 |
|  | 1.00E+08 | 5.00E+06 | 5 | 21 | 27 | 30 |  |  |  |
|  | 1.00E+08 | 5.00E+06 | 6 | 21 | 27 | 26 |  |  |  |
| S3 | 1.00E+08 | 5.00E+06 | 7 | 21 | 27 | 20 | 94 |  |  |

These results strongly support the validity of the proposed model, with its present calibration factors, so that no reason exists up to this point to invalidate the model here developed.

Figure 7.1 – Analysis of Road Note 29 case

# 5 – Concluding Remarks

Key components of a Pavement Management System (PMS) are: the performance prediction models, the decision trees for maintenance needs detection, the resources added as a help in rehabilitation designs at the project level and the production and evaluation of the consequences of maintenance strategies at the network level. Based on 12 years of experience with this PMS, applied to more then 8000 km of highways, the present features of the system can be considered adequate for the multiple functions it has been performing.

The pavement performance models here developed have the feature of using the same model structure regardless of the pavement case where it can be applied (asphalt pavements, rehabilitated ones, rigid pavements and PCC overlaid pavements).

Our best models are only crude representations of reality. The model here developed is no exception. Its intended purpose is to serve as tool for performance prediction at a PMS, for which the model must be adequately calibrated using field data from the monitoring subsystem of the PMS. The present structure of the model, together with the calibration factors here calculated, constitute a reliable tool for Brazilian highway conditions and it is expected that other values for the calibration factors will be required under different climatic, traffic and materials conditions.

# References

AASHTO (1986). The AASHTO Guide for Design of Pavement Structures. American Association of State Highway and Transportation Officials, Washington, DC.

DNIT (1996). Manual de Pavimentação. Publicação IPR-719. Rio de Janeiro, Brasil.

George, K.P. and Rajagopal, A.S. (1989). Life Expectancy of Asphalt Overlays. Second International Symposium on Pavement Evaluation and Overlay Design, Rio de Janeiro. Vol. II, pp. 4.9.1 - 4.9.19.

Paterson, W.D.O. (1987). Road Deterioration and Maintenance Effects: models for planning and management. The World Bank, Washington, DC.

Pereira, A.M. (1979). "Considerações sobre o procedimento de avaliação objetiva da superfície de pavimentos flexíveis e semi-rígidos normalizado pelo DNER. Fundamentos metodológicos. Algumas modificações possíveis". Simpósio Internacional de Avaliação de Pavimentos e Projeto de Reforço, Anexos, Rio de Janeiro, 9 a 14 de setembro, ABPv.

The World Bank (1987). Description of the HDM-III Model. Volume I. Washington, DC.